| Written by: |  | Sign: |  | Rev. |
| --- | --- | --- | --- | --- |
| Paul Connelly |  |  |  | 04-2019 |

Equipment (underline) : SKIPPER DL2 / DL21

| Harbour test: | Status |
| --- | --- |
| - Check the Installation Documentation verify that the correct information is entered |  |
| - Switch on log, adjust brightness & contrast to correct level and to verify function ok. |  |
| - Check function buttons are working and values are shown. |  |
| - Go to diagnostics page and run self diagnostics |  |
| - Activate manual speed and adjust to 10 knots. Verify that repeaters are displaying correct speed & distance. |  |
| - Check actual water temp. with known source and compare with DL readout. |  |
| - Verify that test speed is displayed on radars, conning and ecdis if connected. |  |
| - Verify that the software version of the display and the JB70 unit are matching |  |
| # Comments: |  |
| # Estimated duration: 0,5 Hour |  |
| Tests performed at sea: | Status |
| - Verify proper operation of log. |  |
| - Calibrate log at at least 1 high speed (prefereably high and low speed) in a depth of no greater than 80m |  |
| -Enter the installation angle of the system and verify transversal speed is realistic |  |
| - Verify that the GPS speed on a stable normal operational speed and stable course for a period of 5 minutes produces a distance within 2% of the actual travelled distance. Or by direct comparison of SL SOG and GPS SOG within 2% Make sure the reference distance has at least 2 decimals (1.00) |  |
| # Comments: |  |
| # Estimated duration: 2-4 hours |  |

| OWNER:(sign/date) | CLASS:(sign/date) | YARD:(sign/date) | Resp:(sign/date) |
| --- | --- | --- | --- |